Yttrium (III) chloride hexahydrate 3841 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/yttrium-iii-chloride-hexahydrate-prochem-en-4b3a13b7f6

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Yttrium (III) chloride hexahydrate

PRODUCT NUMBER:  3841

CAS NUMBER:        10025-94-2

SYNONYMS:           Yttrium trichloride hexahydrate

MANUFACTURER:

Prochem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                     TOLL FREE:    800-795-8788

Rockford, IL  61109

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315              Causes skin irritation.

H319              Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338     IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

P362               Take off contaminated clothing and wash before reuse.

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 0

YTTRIUM (III) CHLORIDE HEXAHYDRATE                                                      PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: Causes serious eye irritation.

SKIN CONTACT: Causes skin irritation.

INHALATION: Inhalation of dust can lead to irritation of the respiratory tract.

INGESTION: May be irritating if inhaled.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: YCl3 ! 6H2O

Molecular Weight: 303.36

CHEMICAL NAME                                           CAS#                               %

Yttrium (III) Chloride Hexahydrate                                          10025-94-2                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under eyelids. A

victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if irritation

persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as difficulty in

breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use water spray, alcohol-resistant foam, dry chemical, or carbon dioxide.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen chloride gas and yttrium oxides.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and keep in suitable, closed containers for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Avoid formation of dust.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store in a tightly sealed container.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS:

Component                    Exposure Limits            Basis                          Entity

Yttrium (III) Chloride Hexahydrate    1 mg/m3              TWA                    ACGIH

1 mg/m3              TWA                    NIOSH

YTTRIUM (III) CHLORIDE HEXAHYDRATE                                                      PAGE 2

Page 3

TWA: Time Weighted Average over 8 hours of work.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White powder or crystals

ODOR: None

MOLECULAR WEIGHT: 303.36

BOILING POINT: No data available.

MELTING POINT: 100°C

SPECIFIC GRAVITY: 2.18

VAPOR DENSITY: No data available

SOLUBILITY IN WATER: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: None.

INCOMPATIBILITY: Strong oxidizing agents

DECOMPOSITION PRODUCTS: Hydrogen chloride gas and yttrium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Intraperitoneal (mouse); LD50: 158 mg/kg. Oral (rat); TDLo: 28 gm/kg/28D-1. LD50 Oral – Rat – female - >=2,000

mgkg. LD50 Dermal – Rat – male and female - > 2,000 mg/kg

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: ZG3160000

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

Toxicity to fish semi-static test LC50 - Oncorhynchus mykiss (rainbow trout) - 0.62. mg/l - 96 h

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good

YTTRIUM (III) CHLORIDE HEXAHYDRATE                                                      PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 302/304: Not listed

SARA 311/312: Acute Health Hazard

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE REVISED: 07/26

REVISION SUMMARY: Fax number removed. Updated the following Sections #11 - #12. All other details in the document about the

product remain unchanged. – Revised by EHS 7/26

Reviewed/Approved 6/26

SDS DEPT.

YTTRIUM (III) CHLORIDE HEXAHYDRATE                                                      PAGE 4