Yttrium (III) Nitrate Tetrahydrate 3858 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/yttrium-iii-nitrate-tetrahydrate-prochem-en-2ad3981daf

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Yttrium (III) Nitrate Tetrahydrate

PRODUCT NUMBER:  3858

CAS NUMBER:       13773-69-8

SYNONYMS:           Nitric Acid, yttrium (3+) salt, tetrahydrate

MANUFACTURER:

ProChem, Inc.                                         PHONE:       815-398-1788

826 Roosevelt Road                                     TOLL FREE:    800-795-878

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H272             May intensify fire; oxidizer.

H315              Causes skin irritation.

H319              Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P210              Keep away from heat.

P220                 Keep/store away from clothing/combustible materials.

P221               Take any precaution to avoid mixing with combustibles.

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338     IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P362               Take off contaminated clothing and wash before reuse.

P370+P378             In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

YTTRIUM (III) NITRATE                                                                  PAGE 1

Page 2

P403+P233           Store in a well-ventilated place.

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 2

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 2       Specific Hazard: OX

EYE CONTACT: Causes slight to mild irritation of the eyes.

SKIN CONTACT: Causes slight to mild irritation of the skin.

INHALATION:  Irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Y(NO3)3  4H2O

Molecular Weight: 346.98

CHEMICAL NAME                                           CAS#                              %

Yttrium (III) Nitrate Tetrahydrate                                         13773-69-8                                               100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under eyelids. A

victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if irritation

persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial respiration.

Get medical attention if necessary.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or foam. Water may be ineffective but may be used for cooling

exposed containers.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Nitrogen oxides and yttrium oxides.

SECTION 6  ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid dust

formation or breathing dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with vermiculite or sodium

carbonate and swept up and placed in proper container for disposal.

SECTION 7  HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Keep way from sources of ignition-No smoking. Keep away from heat.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Hygroscopic. Store away from flammable

substances, reducing agents, organic materials, metal powders, water, and moisture. Store under dry inert gas. Protect from humidity

and water.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

YTTRIUM (III) NITRATE                                                                  PAGE 2

Page 3

EYE PROTECTION: Wear chemical safety glasses or goggles.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Off-white solid

ODOR: None

MOLECULAR WEIGHT: 346.98

BOILING POINT: Not available

MELTING POINT: Not available

SPECIFIC GRAVITY: 2.68

VAPOR DENSITY: No data available

SOLUBILITY IN WATER: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Contact with strong reducing agents or organic matter. Some nitrates can explode if heated to high

temperatures.

INCOMPATIBILITY: Flammable substances, reducing agents, water, moisture, organic materials, metal powders, and strong

acids.

DECOMPOSITION PRODUCTS: Nitrogen oxides and yttrium oxides

SECTION 11 TOXICOLOGICAL DATA

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: ZG3750000

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN1477                                                Marine Pollutant: No

Nitrates, inorganic, n.o.s.

(Yttrium (III) Nitrate)

CLASS 5.1

PG II

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Reactivity hazard, acute health hazard, chronic health hazard

YTTRIUM (III) NITRATE                                                                  PAGE 3

Page 4

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc. makes

no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the product based

on the information.

DATE PREVIOUSLY PREPARED: 11/24

DATE REVISED 07/26

REVISION SUMMARY: Removed fax number. All other details in this document about the product remained unchanged. Updated by

EHS on 07/17/26.

EHS/SDS DEPT.

YTTRIUM (III) NITRATE                                                                  PAGE 4